LocalLife Waste Monitoring System

AI Build Playbook for RealSense-Based Bag Volume and Color Estimation

Instruction manual for ChatGPT Cloud, Codex, or any LLM agent building the project from scratch

Build Direction From Latest Meeting Use Intel RealSense as the only authority for geometry, depth, dimensions, mesh, and volume. Do not average RealSense volume with Logitech or monocular-depth volume. Logitech can remain only as a secondary visual sensor for color, texture, material cues, and object-presence confirmation.

# 1. Project Objective

Bag-only scope: the detector must count and measure waste bags only, while rejecting shoes, boxes, bottles, and other non-bag objects.

Counting output: maintain a running session counter of accepted bag events: Bag 1, Bag 2, Bag 3, and so on.

Build a working prototype that estimates the volume and color of individual waste bags or similar deposits using a fixed camera setup. The system must produce a final liter estimate, not only a mesh or a pending status.

Primary output: estimated bag volume in liters from RealSense depth data.

Secondary output: bag color and optional material/texture cues.

Validation output: actual vs estimated volume, percentage error, failure reason, and confidence.

Thesis output: documented method, experiments, error analysis, limitations, and future work.

# 2. Scope Freeze

| Area | Use Now | Do Not Do Now / Future Work |
| --- | --- | --- |
| Volume estimation | RealSense-only depth geometry | Do not average with Logitech or AI monocular depth |
| Object detection | Keep YOLO; use it to find bag/object ROI | Do not restart with a full supervised volume model |
| Segmentation | Use YOLO segmentation if available; otherwise SAM/GrabCut fallback | Do not let poor masks silently produce volume |
| Color/material | Use RealSense RGB first; Logitech optional secondary stream | Do not depend on Logitech for metric scale |
| Deployment | Local laptop or Raspberry Pi edge with optional cloud inference | Do not require manual cloud-only workflow for demo |
| Experiment | Controlled room/bin validation with known volumes | Do not wait for perfect BRF deployment access |

# 3. Hardware Architecture

Intel RealSense D435/D435i: depth + RGB stream, aligned frames, metric depth in meters.

Logitech C920/Brio or similar webcam: optional color/texture support only.

Raspberry Pi 5 or laptop: acquisition, lightweight inference, dashboard, and logging.

Stable mount: fixed height, fixed angle, repeatable field of view.

Lighting: stable indoor lighting; avoid strong reflections and direct sunlight.

Optional sensors: PIR/ToF only for event trigger, not for final volume unless explicitly studied.

# 4. Camera Placement Rule

Mount the RealSense above or slightly oblique to the measurement area so the full bag and the floor/baseline plane are visible. Keep the setup fixed during all experiments. If the camera moves, baseline and plane fitting must be repeated.

Important Calibration is not optional if the camera geometry changes. The latest meeting identifies calibration and cross-camera mismatch as the main reason volume remains pending or wrong.

# 5. Software Stack

| Layer | Library | Purpose |
| --- | --- | --- |
| Camera | pyrealsense2 | Read RealSense RGB/depth, depth scale, intrinsics, frame alignment |
| Image processing | opencv-python | Frame capture, color conversion, masks, contours, morphology |
| Numerics | numpy | Depth arrays, pixel-wise height maps, area/volume integration |
| Geometry | scipy / open3d optional | Plane fitting, point cloud cleanup, mesh/visual QA |
| Detection | ultralytics YOLO | Object/bag detection and optional segmentation mask |
| Segmentation fallback | SAM 2 / Segment Anything / GrabCut | Refine object mask when YOLO only gives boxes |
| Depth QA | RealSense SDK filters | Hole filling, temporal/spatial filtering, invalid-depth handling |
| Dashboard/API | FastAPI + uvicorn | Local inference service and status endpoint |
| UI | Streamlit or lightweight HTML/JS | Live preview, readings table, export button |
| Data logging | pandas + CSV/JSON | Experiment logs, actual vs estimated results |
| Plots | matplotlib/seaborn | Error charts for thesis report |

# 6. Recommended Repository Structure

app/ bag_counter.py # running bag-event counter and duplicate prevention class_filter.py # allow waste_bag only; reject non-bag objects

locallife_volume/ app/ main.py # FastAPI entrypoint config.py # camera height, ROI, thresholds, paths camera_realsense.py # RealSense capture and aligned RGB/depth camera_logitech.py # optional RGB-only capture detector_yolo.py # YOLO detection/segmentation wrapper segmentation.py # mask refinement and fallbacks depth_geometry.py # floor plane, height map, volume integral color_material.py # dominant color and simple material cues tracking.py # object stability and reading trigger confidence.py # flags for invalid depth / bad mask / motion logger.py # CSV/JSON experiment logging dashboard/ streamlit_app.py # preview, controls, readings, export data/ baselines/ readings/ exports/ models/ yolo_bag_seg.pt tests/ test_volume_math.py test_depth_units.py test_mask_area.py README.md

# 7. End-to-End Pipeline

Start RealSense stream and align depth frame to RGB frame.

Capture empty baseline frame or estimate floor plane from visible floor/bin-bottom pixels.

Run YOLO on the RealSense RGB frame to detect the bag/object ROI.

Create a clean segmentation mask for the object. Prefer YOLO segmentation; fallback to SAM or OpenCV mask refinement.

Extract the RealSense depth pixels inside the mask.

Remove invalid depth pixels, holes, and extreme outliers using RealSense filters and NumPy percentiles.

Convert depth into a height map relative to the empty floor/baseline plane.

Integrate height over pixel-projected area using RealSense intrinsics.

Convert cubic meters to liters.

Compute confidence flags and reject readings that are too noisy, too reflective, too small, or moving.

Estimate dominant color from masked RGB pixels.

Save reading with timestamp, actual volume if known, estimated volume, error, mask area, confidence, and notes.

# 8. Core Volume Mathematics

Use direct geometric calculation from metric RealSense depth. Do not train an image-to-volume regressor for the current thesis build.

Inputs: D_after(u, v) = RealSense metric depth after object is present, in meters D_floor(u, v) = empty floor/bin baseline depth at the same pixel, in meters M(u, v) = object segmentation mask, 1 inside object and 0 outside fx, fy = RealSense focal lengths in pixels Per-pixel projected surface area approximation: A_pixel(u, v) = D_after(u, v)^2 / (fx * fy) Height above floor/baseline: H(u, v) = max(0, D_floor(u, v) - D_after(u, v)) Volume: V_m3 = sum over mask pixels: M(u, v) * H(u, v) * A_pixel(u, v) V_liters = V_m3 * 1000

Plane-Based Improvement If the camera or floor is tilted, replace the simple per-pixel baseline depth with a fitted 3D floor plane. Convert valid depth pixels to 3D points, fit the floor plane from baseline/floor pixels, compute object height as distance above the plane, then integrate.

# 9. RealSense Depth Processing Requirements

Use rs.align to align depth to RGB before using YOLO masks.

Use the camera depth scale and keep all distances in meters until the final liters conversion.

Apply spatial, temporal, hole-filling, and decimation filters only if they improve stability without deforming object shape.

Reject pixels with depth equal to zero or outside a realistic range.

Use median depth and interquartile range to detect unstable frames.

For black/dark bags, expect holes/noise; report this limitation and use confidence flags.

# 10. YOLO Module

YOLO is considered sufficient in the latest meeting. The build should not spend time replacing it unless object detection clearly fails.

from ultralytics import YOLO model = YOLO("models/yolo_bag_seg.pt") # segmentation model preferred result = model(rgb_frame, imgsz=640, conf=0.25)[0] # Output needed by geometry module: # - bounding box # - class label # - confidence # - binary mask aligned with RealSense RGB/depth frame

If YOLO only returns bounding boxes, use the box as a prompt for SAM or an OpenCV segmentation fallback.

Do not batch unsynchronized RealSense and Logitech frames for volume. If batching is used, batch only frames that are safe for the same stream/task.

For moving/falling bags, only measure after object stability is detected across multiple frames.

# 11. Segmentation and Mask Cleanup

| Problem | Required Handling |
| --- | --- |
| Mask includes floor/background | Erode/dilate, largest connected component, remove pixels with near-floor height |
| Mask misses folded bag parts | Use SAM refinement or lower YOLO mask threshold |
| Depth holes inside mask | Fill small holes; reject frame if missing-depth ratio is high |
| Multiple objects | Measure largest object or log separate objects only if segmentation is reliable |
| Reflective/shiny object | Flag as unreliable; do not force a fake liter value |

# 12. Color and Material Module

Color should be calculated from masked RGB pixels, not the full frame. RealSense RGB can be the default. Logitech may be used only when synchronized enough for visual labeling and when its field of view sees the same object.

Convert masked RGB pixels to HSV or LAB color space.

Ignore overexposed, shadowed, and very dark pixels when estimating dominant color.

Return a simple label: black, white, grey, blue, green, yellow, red, brown, transparent/unknown.

Optional material cues: glossy/reflection score, texture variance, transparency cue, but keep it secondary.

# 13. Confidence and Failure Logic

| Flag | Trigger | System Action |
| --- | --- | --- |
| LOW_MASK_CONFIDENCE | YOLO/SAM confidence low or mask too small | Do not output final volume |
| DEPTH_HOLES_HIGH | Large percentage of zero/invalid depth inside mask | Retry or flag reflective/dark issue |
| OBJECT_MOVING | Mask centroid/area changes over consecutive frames | Wait until stable |
| BASELINE_MISSING | No empty baseline or no fitted plane | Request baseline capture |
| VOLUME_OUTLIER | Volume jumps too much across stable frames | Use median of stable frames or reject |
| REFLECTIVE_OBJECT | Depth failure on shiny material | Report unsupported/fallback required |

# 14. Deployment Plan

## 14.1 Local Laptop Build

python -m venv .venv source .venv/bin/activate pip install pyrealsense2 opencv-python numpy scipy pandas matplotlib seaborn fastapi uvicorn ultralytics open3d # Optional segmentation fallback pip install torch torchvision transformers segment-anything uvicorn app.main:app --host 0.0.0.0 --port 8200 streamlit run dashboard/streamlit_app.py --server.port 8501

## 14.2 Raspberry Pi Build

Use Pi primarily for capture, event trigger, and lightweight inference if performance allows.

If YOLO is slow, run capture on Pi and inference on laptop/cloud through a local API.

Keep RealSense on USB 3.0 and verify stable frame rate.

Always log raw RGB/depth snapshots for failed trials so debugging is possible.

# 15. API Contract

| Endpoint | Purpose | Response |
| --- | --- | --- |
| GET /counter/status | Read current bag counter and active tracks | running_bag_count, active_track_ids |
| POST /counter/reset | Start a fresh experiment session | running_bag_count=0, tracks_cleared=true |

| Endpoint | Purpose | Response |
| --- | --- | --- |
| GET /health | Check camera/model/dashboard status | camera_connected, model_loaded, version |
| POST /baseline/capture | Capture empty baseline / fit floor plane | baseline_id, timestamp, quality |
| POST /measure | Run one stable measurement | volume_liters, color, confidence, flags |
| GET /stream | Preview frames and overlays | RGB + mask + depth visualization |
| POST /reading/groundtruth | Attach actual volume for validation | reading_id, actual_liters, error_percent |
| GET /export.csv | Export experiment table | CSV file |

# 16. Validation Protocol for Thesis

A 20-25 reading set is acceptable only as a controlled pilot if it is stratified and includes ground truth. A stronger thesis should target 40-50 readings if time allows.

| Test Group | Minimum Count | Purpose |
| --- | --- | --- |
| Known boxes/containers: 1L, 2L, 5L, 10L | 8-10 | Verify units and integration math |
| Waste bags with measured/known fill volume | 12-15 | Main thesis dataset |
| Different colors/materials | 5-8 within above | Color and depth robustness |
| Off-center/tilted placements | 3-5 within above | Placement sensitivity |
| Failure cases: glossy/shiny, very dark, moving | 3-5 | Limitations and confidence flags |

For each trial, record actual volume, estimated volume, color, mask quality, depth quality, setup height, setup angle, lighting, and notes.

Use median absolute percentage error, mean absolute percentage error, 90th percentile error, and failure rate.

Report success separately from rejected/low-confidence readings. A rejected bad reading is better than a fake precise number.

# 17. Expected Thesis Results

| Result Item | Target Output |
| --- | --- |
| Volume accuracy | Median percentage error for controlled bags/objects |
| Color performance | Simple accuracy/count of correct dominant color labels |
| System reliability | How often a reading becomes final vs pending/rejected |
| Failure mode analysis | Dark bags, reflective objects, motion blur, bad mask, floor tilt |
| Architecture conclusion | RealSense-only geometry is the recommended MVP |
| Future work | Logitech material module, monocular AI comparison, larger BRF deployment |

# 18. Do Not Build These in the Current Sprint

Do not train a supervised image-to-volume model from 500+ images for the immediate deadline.

Do not average Logitech depth with RealSense depth.

Do not treat Depth Anything V2 relative depth as metric depth without calibration.

Do not measure a falling bag unless the explicit experiment is motion blur/in-flight measurement.

Do not show volume as pending forever; return a clear reason code and debugging data.

# 19. AI Builder Master Prompt

Copy this section into ChatGPT Cloud, Codex, or another LLM builder.

Build the LocalLife waste-monitoring prototype according to this architecture: 1. Use Intel RealSense as the only source for depth, geometry, dimensions, mesh, and volume. 2. Do not use Logitech for volume and do not average camera volumes. 3. Keep YOLO for bag/object detection. Use YOLO segmentation if available; otherwise use YOLO box plus SAM/OpenCV mask refinement. 4. Align RealSense depth to RealSense RGB, then apply the object mask to the aligned depth frame. 5. Capture an empty baseline or fit a floor/bin-bottom plane. Compute object height as the difference between baseline/floor plane and object surface depth. 6. Calculate volume by summing pixel-wise columns inside the mask using RealSense intrinsics: V_m3 = sum(M * H * D^2 / (fx * fy)); V_liters = V_m3 * 1000. 7. Add depth filtering, invalid-pixel rejection, mask cleanup, object-stability checking, and confidence flags. 8. Color must be estimated from masked RGB pixels. Logitech may only assist color/texture if needed. 9. Provide a local dashboard with RGB/depth/mask overlay, volume in liters, color, confidence flags, and CSV export. 10. Implement experiment logging for actual volume vs estimated volume and compute percentage error. 11. Validate first with known-volume objects and then waste bags in a controlled room/bin setup. 12. Treat glossy/reflective objects as an explicit limitation/failure mode unless reliable depth exists. Deliver working code, a README, a sample CSV schema, and a short test plan. The final system must never leave the volume stuck at 'pending' without a reason code.

# 20. Acceptance Checklist

Non-bag objects are rejected and do not increase the running bag counter.

The same bag is not counted repeatedly while it remains in the camera view.

The dashboard shows running_bag_count and the latest accepted bag_instance_id.

RealSense RGB and depth streams are aligned and visible.

Empty baseline or floor plane can be captured and quality-checked.

YOLO detects the bag/object and produces a usable mask.

Volume module returns liters for known objects with reasonable error.

Dashboard shows final volume or a clear rejection reason.

Logitech is not used in the volume calculation.

CSV export contains all required thesis fields.

At least 20-25 stratified readings are collected with actual volume labels.

Error charts and failure-case examples can be generated from the logs.

README explains setup, calibration/baseline, run command, and validation protocol.

# 21. Minimal Experiment CSV Schema

| Column | Example | Reason |
| --- | --- | --- |
| trial_id | BAG_001 | Unique trial reference |
| timestamp | 2026-09-07 18:30 | Traceability |
| object_type | waste_bag | Class/category |
| bag_color_actual | black | Manual label |
| bag_color_predicted | black | System output |
| actual_volume_liters | 10.0 | Ground truth |
| estimated_volume_liters | 11.4 | Main result |
| absolute_error_liters | 1.4 | Error magnitude |
| percentage_error | 14.0 | Thesis metric |
| mask_area_pixels | 45231 | Debugging |
| invalid_depth_ratio | 0.08 | Depth quality |
| confidence | 0.82 | Reliability |
| flags | none | Reason if rejected |
| notes | slightly folded | Qualitative context |

# 22. Bag-Only Detection and Running Counter

This project must be scoped as a bag-only waste-monitoring prototype. The system should detect, count, measure, and log waste bags or garbage bags only. It must not become a general object-volume system.

Core rule The running counter means total valid bag events processed during the session: Bag 1, Bag 2, Bag 3, and so on. It is not the number of objects currently visible in one frame.

## 22.1 Positive and Negative Scope

| Valid Target Objects | Must Be Ignored / Negative Objects |
| --- | --- |
| Plastic waste bags and garbage bags | Shoes, sandals, clothes, books, tools |
| Soft deformable trash bags | Cardboard boxes, cartons, paper stacks |
| Different bag colors: black, blue, white, yellow, grey, green | Bottles, cans, metal items, glossy containers |
| Bags placed on the floor or inside the test bin | Hands, body parts, furniture, random background objects |
| Single bags and multiple separate bags if masks are separable | Any non-bag object even if volume can be estimated |

## 22.2 YOLO Class Strategy

The production detector should expose one accepted class: waste_bag. Synonyms such as garbage_bag, trash_bag, and plastic_bag should be mapped internally to waste_bag.

Training or fine-tuning data must include negative/background examples: shoes, boxes, bottles, hands, books, and other household objects. These objects should not be labeled as valid classes.

If a generic pretrained model detects bottle, box, shoe, person, or other classes, these classes must be filtered out before counting and volume estimation.

The system should return ignored_object for non-bag detections and leave running_bag_count unchanged.

## 22.3 Counting Logic

A bag count should be added only after a valid bag is detected, remains stable, passes quality checks, and has not already been counted.

| State | Meaning | Next Action |
| --- | --- | --- |
| IDLE | No valid bag candidate in the measurement area. | Wait for a waste_bag detection. |
| CANDIDATE | A possible waste bag is detected. | Track for several frames and check mask/depth quality. |
| STABLE_VALID_BAG | The same bag is stable and passes thresholds. | Assign the next bag number and measure volume. |
| COUNTED | The bag has already been counted. | Do not increment again while it remains visible. |
| WAIT_CLEAR | The system waits for the scene to clear or a new deposit event. | Return to IDLE when object leaves or new event starts. |
| IGNORED_OBJECT | A non-bag object is detected. | Log as negative test; do not count or measure as bag. |

## 22.4 Duplicate Prevention Rules

Use object tracking based on mask centroid, bounding box overlap, and mask area similarity across frames.

Require stability for at least 5-10 consecutive frames before assigning a new bag count.

Use a cooldown period after counting so the same stationary bag is not counted again.

If multiple bags are visible and separable, assign separate bag_instance_id values. If they merge into one mask, log as merged_bag_group and report combined volume with a lower confidence flag.

Provide a reset counter control at the start of each experiment session.

## 22.5 Example Expected Output

| Event | Running Count | Detected | Decision | Color | Volume (L) | Status / Reason |
| --- | --- | --- | --- | --- | --- | --- |
| E01 | 1 | waste_bag | Valid bag | Black | 10.8 | Counted as Bag 1 |
| E02 | 2 | waste_bag | Valid bag | Blue | 14.2 | Counted as Bag 2 |
| E03 | 2 | shoe | Rejected | N/A | N/A | Ignored; counter unchanged |
| E04 | 2 | cardboard_box | Rejected | N/A | N/A | Outside thesis scope |
| E05 | 3 | waste_bag | Valid bag | White | 8.7 | Counted as Bag 3 |
| E06 | 4 | garbage_bag | Valid bag | Yellow | 12.4 | Mapped to waste_bag |
| E07 | 4 | bottle | Rejected | N/A | N/A | Non-bag object |
| E08 | 5 | waste_bag | Valid bag | Black | 16.1 | Counted as Bag 5 |

## 22.6 Implementation Pseudocode

running_bag_count = 0 tracked_objects = {} for frame in realsense_stream: detections = yolo.detect(frame.rgb) bag_candidates = filter(detections, class_name in ["waste_bag", "garbage_bag", "trash_bag"]) ignored_objects = filter(detections, class_name not in accepted_bag_classes) log_ignored_objects(ignored_objects, counter_unchanged=True) for candidate in bag_candidates: mask = get_or_refine_mask(candidate) if not passes_mask_depth_stability_checks(mask, frame.depth): log_rejected_candidate(candidate, reason_code) continue track_id = match_to_existing_track(candidate, tracked_objects) if track_id is new and stable_for_required_frames(candidate): running_bag_count += 1 bag_number = running_bag_count volume_liters = estimate_realsense_volume(mask, frame.depth, baseline_or_plane) color = estimate_masked_color(frame.rgb, mask) log_counted_bag(bag_number, volume_liters, color) else: keep_tracking_without_incrementing_counter()

## 22.7 Result Logging Fields for Counting

| Field | Meaning |
| --- | --- |
| event_id | Sequential event or frame-group ID, e.g., E001 |
| running_bag_count | Total number of accepted bags counted so far |
| bag_instance_id | Unique ID for each accepted bag, e.g., BAG_001 |
| detected_class_raw | Raw detector label before filtering |
| accepted_class | waste_bag only, or empty if rejected |
| valid_bag | true only if object is accepted as a waste bag |
| counter_incremented | true only on the first accepted event for a new bag |
| duplicate_track | true if same bag remains visible and must not be recounted |
| ignored_object_type | shoe, box, bottle, etc. for negative tests |
| estimated_volume_liters | Volume from RealSense geometry for accepted bags only |
| bag_color_predicted | Dominant bag color from masked RGB |
| status | counted, ignored, rejected, pending_with_reason |
| reason_code | non_bag_object, low_mask_confidence, depth_holes_high, duplicate_track, moving_object |

## 22.8 Counter APIs and Dashboard Requirements

| Feature | Requirement |
| --- | --- |
| Dashboard counter | Show Total Bags Counted in the current session. |
| Counter reset | Provide a reset button before a new experiment run. |
| Counter status API | GET /counter/status returns running_bag_count and active tracks. |
| Counter reset API | POST /counter/reset clears count and active tracks. |
| Negative object log | Ignored objects must appear in logs but not in counted-bag totals. |
| Export | CSV must include both accepted bag events and ignored negative tests. |

# 23. Final Recommendation

The fastest defendable thesis path is not to rebuild the full system around AI monocular depth. The current project should be built around a RealSense-only geometric volume baseline, with YOLO/SAM for object masking and simple color/material outputs as secondary functions. Once this works, collect stratified known-volume readings and report accuracy, reliability, and limitations honestly.